🧭 Visual Flow

Here’s how a typical request flows through the OSI layers:

- User clicks a button in React → triggers an HTTP request.

- Application Layer formats the request (HTTP).

- Presentation Layer encrypts it (HTTPS).

- Session Layer manages the connection (maybe keeps it alive).

- Transport Layer ensures it gets there (TCP).

- Network Layer routes it to Azure (IP).

- Data Link + Physical Layers handle the actual transmission.

🧩 OSI Layers Present on the Client Side

Even though you're just writing frontend code, your computer still uses all 7 layers of the OSI model when sending or receiving data over the internet.

Let’s walk through them using a real-world example:
You click a button in your React app that sends a request to a .NET API hosted on Azure.

✅ Layers Active on Your Client Machine

| Layer | Role on Client | Example in React/Web |
| --- | --- | --- |
| 7. Application | Your browser or React app initiates the request | fetch('/api/data') or axios.get(...) |
| 6. Presentation | Formats data (e.g., JSON) and encrypts it | HTTPS, JSON.stringify() |
| 5. Session | Manages connection state | TLS handshake, WebSocket session |
| 4. Transport | Ensures reliable delivery | TCP ensures packets arrive correctly |
| 3. Network | Routes data to the server | IP address of Azure server |
| 2. Data Link | Sends data to your router/modem | MAC address, Ethernet/Wi-Fi |
| 1. Physical | Moves bits over cables or wireless | Fiber, Wi-Fi, 4G/5G |

🧠 What You Actually Interact With

As a web developer, you mostly interact with:

- Layer 7 (Application): HTTP, Web APIs, browser.

- Layer 6 (Presentation): JSON, HTTPS.

- Layer 5 (Session): WebSockets, cookies, tokens.

- Layer 4 (Transport): You don’t touch TCP directly, but it powers HTTP.

The lower layers (3 to 1) are handled by your operating system, browser, and network hardware.

🧪 Example Flow: React → .NET API

- React app calls fetch('/api/data').

- Browser wraps it in an HTTP request (Layer 7).

- Data is encrypted with TLS (Layer 6).

- A TCP connection is used (Layer 4).

- The request is routed via IP to Azure (Layer 3).

- Sent over Wi-Fi/Ethernet (Layer 2).

- Transmitted as electrical or radio signals (Layer 1)

🧠 OSI Model Explained with React + .NET + Azure

🧱 Layer 7: Application Layer

- What it does: Provides services directly to the user.

- In your app: React app makes HTTP requests to your .NET API.

- Example: fetch('/api/data') or axios.get('/api/data').

🧠 Layer 6: Presentation Layer

- What it does: Formats and encrypts data.

- In your app: JSON formatting, HTTPS encryption.

- Example: Your API returns JSON, and HTTPS ensures secure transmission.

🧠 Layer 5: Session Layer

- What it does: Manages sessions and persistent connections.

- In your app: WebSocket connections or session tokens.

- Example: JWT tokens for user sessions, or WebSocket for live chat.

🧠 Layer 4: Transport Layer

- What it does: Ensures reliable delivery of data.

- In your app: TCP is used under the hood for HTTP/HTTPS.

- Example: Azure ensures your API calls are reliably delivered using TCP.

🧠 Layer 3: Network Layer

- What it does: Routes data across networks.

- In your app: IP addresses and routing between client and Azure servers.

- Example: Your React app hosted on a user's browser sends requests to an Azure IP.

🧠 Layer 2: Data Link Layer

- What it does: Transfers data between devices on the same network.

- In your app: Handled by routers/switches in Azure and the user's ISP.

- Example: Not directly visible, but crucial for packet delivery.

🧠 Layer 1: Physical Layer

- What it does: Moves bits over cables or wireless.

- In your app: Fiber optics, Wi-Fi, Ethernet.

- Example: The actual internet connection used by the user and Azure data center.